Single Choice Questions

1. What is the function of the direction cosine matrix Cbn? (B)

A. Represents the angular velocity of the vehicle.

B. Transforms vectors from the body frame to the navigation frame.

C. Calculates the acceleration of the vehicle.

D. Represents the curvature of the Earth.

2. In the numerical update algorithm for dead reckoning, using the average of the "previous velocity" and the "current velocity" for integration essentially adopts which numerical integration method? (C)

A. Runge-Kutta method

B. Euler method

C. Trapezoidal rule

D. Newton's method

3. In dead reckoning, the rate of altitude change is related to which velocity component? (C)

A. v_N

B. v_E

C. v_D

D. v_x_b

4. In the theoretical trajectory model for the launch vehicle provided in the course materials, what Earth parameter is its initial position mainly based on? (C)

A. Earth's rotational angular velocity ωie

B. Earth's gravitational constant μ

C. Earth's equatorial radius RE

D. Earth's oblateness J2

5. In the attitude update algorithm based on quaternions, the discretized update formula is typically expressed as? (B)

A. qk+1i=qki+Δt⋅qi

B. qk+1i=qki⊗Δq

C. qk+1i=C(Δθ)qki

D. q

6. In an inertial-based integrated navigation system, which sensor is primarily used to provide absolute position and velocity information? (C)

A. Inertial Measurement Unit (IMU)

B. Doppler Velocity Log (DVL)

C. Global Navigation Satellite System (GNSS)

D. Visual sensor

7. The main difference between Extended Kalman Filter (EKF) and standard Kalman Filter (KF) lies in their handling of what type of system? (C)

A. Linear systems

B. Gaussian noise systems

C. Nonlinear systems

D. Discrete-time systems

8. What is the main purpose of transfer alignment in achieving fast and high-precision alignment for the slave inertial navigation system in a dynamic environment? (C)

A. Autonomous navigation

B. Attitude resolution

C. Fast and high-precision alignment

D. Independent positioning

9. During airborne transfer alignment, what parameters of the master inertial navigation system are typically copied directly to the slave inertial navigation system in the first step? (C)

A. Biases and scale factors

B. Installation error angles and lever arms

C. Attitude, velocity, and position

D. Gyroscope outputs and accelerometer outputs

10. Which of the following is not a common assumption during the coarse alignment phase? (B)

A. The vehicle is stationary.

B. Significant external acceleration disturbances exist.

C. Gyroscope bias errors are neglected.

D. The navigation frame is the local geographic coordinate system (NED).

11. Which of the following is **not** a disadvantage of analytic coarse alignment? (D)

A. Reduced north-seeking accuracy in high-latitude regions.

B. Performance degradation in swaying environments.

C. Susceptibility to accelerometer bias.

D. Extremely high computational complexity, making real-time implementation difficult.

12. When performing fine alignment within the Extended Kalman Filter (EKF) framework, which of the following is typically **not** included in the state vector? (B)

A. Position error.

B. Earth's rotation angle.

C. Attitude error angles.

D. Velocity error.

13. In State Transformation EKF (ST-EKF), what is the main advantage of state transformation? (D)

A. Reduces system noise.

B. Increases sampling frequency.

C. Simplifies hardware design.

D. Improves the observability and consistency of the filter model.

14. In an integrated navigation system, what is the main advantage of State Transformation EKF (ST-EKF) compared to traditional EKF? (C)

A. Reduces computational complexity.

B. Simplifies the filter initialization process.

C. Improves the modeling accuracy of nonlinear systems.

D. Reduces the number of sensors required.

15. In an integrated navigation model, δvn typically represents (D)

A. The absolute value of velocity in the n-frame.

B. Acceleration disturbance in the n-frame.

C. Velocity measurement noise in the n-frame.

D. Velocity error in the n-frame.

16. When using EKF for INS/GNSS integrated navigation, what is the primary source of process noise in the state equation? (C)

A. Satellite clock error.

B. Ionospheric delay.

C. Noise from the IMU's gyroscopes and accelerometers.

D. GNSS receiver's code tracking error.

17. Which of the following integrated navigation modes has the strongest anti-interference capability and highest level of system integration? (D)

A. Loosely coupled INS/GNSS.

B. Non-coupled INS and GNSS operating independently.

C. Tightly coupled INS/GNSS.

D. Deeply coupled (Deep Integration) INS/GNSS.

18. Which of the following is not an advantage of a Deeply Coupled INS/GNSS system? (D)

A. Improved anti-interference capability.

B. Enhanced dynamic performance.

C. Faster satellite signal reacquisition.

D. Reduced hardware requirements for the GNSS receiver.

19. In an unmanned platform's inertial/satellite integrated navigation system, one of the main characteristics of using a loosely coupled architecture is? (C)

A. Directly utilizes raw pseudorange and carrier phase observations.

B. Significantly improves the system's ability to suppress multipath effects.

C. Fuses the position and velocity information output by the GNSS receiver.

D. Can continue to provide high-precision navigation solutions during GNSS signal loss.

20. Which of the following is **not** a typical state variable in EKF for vehicular integrated navigation? (B)

A. Position error.

B. Wheel radius.

C. Velocity error.

D. Attitude error.

21. In long-duration navigation experiments, what is the advantage of SINS/ODO-EKF compared to pure dead reckoning? (A)

A. Significantly reduces the divergence of position error.

B. Improves the accuracy of vehicle speed measurement.

C. Completely eliminates attitude drift.

D. No initial alignment required.

22. In vehicular inertial/odometer integrated navigation, what is the primary measurement provided by the odometer? (D)

A. Angular velocity.

B. Attitude angles.

C. Linear acceleration.

D. Velocity information.

23. What is the main role of the Extended Kalman Filter (EKF) in inertial/odometer integrated navigation? (C)

A. Directly measures attitude angles.

B. Increases the pulse frequency of the odometer.

C. Estimates system state errors and performs correction.

D. Replaces the Inertial Measurement Unit (IMU).

24. In an Autonomous Underwater Vehicle (AUV) inertial/Doppler integrated navigation system, what is the main role of the DVL? (C)

A. Measures the attitude angles of the AUV.

B. Provides absolute position information of the AUV.

C. Measures the AUV's velocity relative to the seabed.

D. Corrects the gravity error of the inertial navigation system.

25. In an AUV integrated navigation system, what type of observation information does USBL primarily provide? (C)

A. Velocity increment.

B. Depth information.

C. Relative position to a surface base station.

D. Heading angle.

26. In an EKF-based AUV integrated navigation system, which of the following is typically not included in the state vector? (C)

A. Attitude error.

B. Velocity error.

C. Acceleration.

D. Gyroscope bias.

27. What is the core concept of the MSCKF algorithm for fusing visual and inertial data? (C)

A. Direct optical flow matching.

B. Single-frame feature point triangulation.

C. Minimization of multi-frame feature reprojection errors within a sliding window.

D. Global SLAM map optimization.

28. In unmanned platform inertial/visual integrated navigation, what is the main role of the IMU? (D)

A. Provides environmental texture information.

B. Recognizes road signs.

C. Detects static obstacles.

D. Measures angular velocity and acceleration.

29. Which of the following feature extraction algorithms does **not** belong to the corner detection category? (B)

A. Harris.

B. SURF.

C. FAST.

D. Shi-Tomasi.

30. What type of visual-inertial navigation method did the Google Tango project primarily adopt? (D)

A. SLAM front-end + graph optimization back-end.

B. Pure visual odometry.

C. Loosely coupled visual/IMU fusion.

D. Tightly coupled filtering method based on MSCKF.

31. In a visual-inertial system, why is time synchronization calibration between the IMU and the camera necessary? (C)

A. To avoid image blur.

B. To reduce IMU noise.

C. To ensure IMU integration results accurately correspond to the image acquisition moments.

D. To improve image resolution.

Multiple Choice Questions

1. In a gimbaled inertial navigation system, which coordinate frames can the stabilized platform typically be stabilized in? (ABC)

A. Inertial Frame

B. Earth Frame

C. Local Navigation Frame

D. Body Frame

2. Which of the following descriptions about DVL (Doppler Velocity Log) are correct? (ABE)

A. Can measure the velocity of an underwater vehicle relative to the seabed.

B. Relies on the acoustic Doppler effect.

C. Cannot work in stationary fluid.

D. Measurement accuracy is unaffected by depth.

E. Typically integrated with INS for underwater navigation.

3. What are the basic steps of transfer alignment? (AE)

A. Coarse alignment: Copying the master INS's position, attitude, and velocity information.

B. Master INS performing autonomous navigation.

C. Eliminating all error sources.

D. Slave INS performing independent initialization.

E. Fine alignment: Estimating residual errors through filtering.

4. Which of the following statements are correct regarding vehicular transfer alignment? (ABC)

A. Changes in heading angle most significantly improve the observability of lever arm errors.

B. Variance suppression can be used to prevent excessive convergence of the vertical accelerometer bias.

C. Changes in roll and pitch angles provide poor observability for the vertical lever arm.

D. The estimation of installation error angles does not depend on the accuracy of the master INS.

5. In the fine alignment phase of transfer alignment, which measurements can be used for filter estimation? (ACD)

A. Velocity difference between the master and slave INS.

B. Angular velocity difference between the master and slave INS.

C. Position difference between the master and slave INS.

D. Attitude difference between the master and slave INS.

6. What are the core factors affecting transfer alignment performance? (ACD)

A. The degree of vehicle maneuver.

B. The sampling frequency of the slave INS.

C. The initial alignment accuracy of the master INS.

D. The modeling accuracy of dynamic flexure deformation.

7. Regarding the differences between Gimbaled INS and Strap-down INS, which of the following statements are correct? (ABC)

A. Gimbaled systems isolate vehicle angular motion through a mechanical stabilized platform.

B. Gimbaled systems have complex structures but lower computational burden.

C. Strap-down systems rigidly mount inertial sensors directly onto the vehicle.

D. Strap-down systems can output navigation information without attitude resolution.

8. Regarding the relationship between coarse and fine alignment, which of the following descriptions are correct? (ABC)

A. Coarse alignment provides an initial attitude estimate for fine alignment.

B. The accuracy of coarse alignment directly affects the convergence speed of fine alignment.

C. Fine alignment typically uses filtering methods to further correct attitude errors.

D. The two can be performed independently, and the order does not matter.

9. Which of the following factors affect the accuracy of analytic coarse alignment? (ABCD)

A. Accelerometer noise.

B. Whether the vehicle is completely stationary.

C. Gyroscope noise.

D. Latitude.

10. In stationary base fine alignment, which can be used as filter measurements? (AC)

A. Zero velocity.

B. Gravitational acceleration.

C. Known position.

D. Earth's rotational angular velocity.

11. Which of the following are typical state variables of EKF in SINS/GPS integrated navigation? (ABDE)

A. Velocity error δv

B. Attitude error angles δϕ

C. GPS clock drift rate

D. Position error δr

E. Gyroscope bias εb

12. Which of the following statements regarding the definition of velocity error are correct? （ABE）

A. The transformed definition is performed in the same navigation frame.

B. The traditional definition is based on the velocity difference in different navigation frames.

C. The two definitions are mathematically completely equivalent.

D. The traditional definition is more suitable for high-dynamic environments.

E. The transformed definition eliminates the influence of specific force terms.

13. Which of the following descriptions about the limitations of the traditional EKF-INS-NED model are correct? (ABE)

A. The system matrix contains specific force terms, making it susceptible to dynamic environmental influences.

B. Heading angle divergence may occur in quasi-static environments.

C. Must rely on GNSS signals.

D. Cannot perform position estimation.

E. Poor adaptability to large misalignment angle initialization.

14. Under large misalignment angle conditions, how does state transformation improve filter performance? (BDE)

A. Introduces additional GNSS observations.

B. Maintains model consistency through nonlinear error variables.

C. Increases the order of gyroscope bias.

D. Decomposes coupled errors using constraint relationships.

E. Avoids the failure of small-angle assumptions.

15. Which of the following factors affect the performance of an INS/GNSS integrated navigation system? （ABDE）

A. GNSS satellite geometry (DOP).

B. IMU sensor accuracy.

C. Environmental temperature.

D. Filter design.

E. Integration mode.

16. Which of the following are common integration levels for GNSS/INS integrated navigation? (ABDE)

A. Loosely coupled.

B. Non-coupled.

C. Fully coupled.

D. Tightly coupled.

E. Deeply coupled.

17. Which of the following factors affect the accuracy of odometer speed measurement? (ABE)

A. Road surface slippage.

B. Tire wear.

C. Insufficient satellite visibility.

D. Excessive vehicle speed.

E. Odometer scale factor error.

18. Which of the following are advantages of INS/Odometer integrated navigation? (ACD)

A. Suppresses the accumulation of INS errors over time.

B. Provides long-term high-precision positioning without external signals.

C. Suitable for urban canyon environments.

D. Provides continuous velocity and attitude output.

E. Completely eliminates gyroscope drift.

19. Which of the following sensors can be used in an underwater integrated navigation system for deep-sea AUVs? (ABDE)

A. Inertial Measurement Unit (IMU).

B. Doppler Velocity Log (DVL).

C. GNSS receiver.

D. Ultra-Short Baseline (USBL) system.

E. Depth sensor.

20. Why is it necessary to consider the lever arm effect in an AUV navigation system? （ABC）

A. Sensors are distributed at different locations.

B. Motion generates velocity differences.

C. Attitude changes cause position offsets.

D. Affects data synchronization between DVL and INS.

E. Causes observation delays.

21. Which of the following factors affect the accuracy of a visual-inertial navigation system? （BDE）

A. Camera exposure time duration.

B. IMU bias stability.

C. Earth's rotational angular velocity.

D. Uniformity of feature point distribution.

E. Environmental lighting changes.

22. Which of the following modules belong to the typical front-end processing pipeline of visual-inertial navigation? （AB）

A. Feature extraction and matching.

B. IMU pre-integration.

C. Kalman filter prediction.

D. Back-end nonlinear optimization.

E. Pose graph construction.

23. Which of the following statements about KLT optical flow are correct? (ABD)

A. Based on the brightness constancy assumption.

B. Suitable for small displacement tracking.

C. Belongs to descriptor matching methods.

D. Can be used for continuous feature point tracking.

E. Must be used in conjunction with FAST corners.

24. In a visual-inertial system, what is RANSAC primarily used to solve? (AB)

A. Estimating the fundamental matrix or essential matrix.

B. Outlier rejection of mismatched feature points.

C. Calibrating IMU installation angles.

D. Optimizing the global pose graph.

E. Selecting the optimal feature subset.

True or False Questions

1. The direction cosine matrix has 9 parameters, but only 3 are independent. (√)

2. Equivalent rotation vectors and quaternions can be converted to each other. (√)

3. In an inertial frame, the Earth's rotational angular velocity ωie has no direct effect on the vehicle's navigation equations. (√)

4. In strapdown inertial navigation, the output of the accelerometer can be directly used as the acceleration term ai in the navigation equations. (×)

5. EKF can be directly applied to strongly nonlinear systems without any approximation. (×)

6. Dynamic flexure deformation angles are usually modeled as a second-order Markov process, but are often simplified and ignored in practice. (√)

7. In the observation model for transfer alignment, the attitude difference can be obtained by directly subtracting Euler angles. (×)

8. When the vehicle is not maneuvering, transfer alignment can still effectively estimate lever arm errors. (×)

9. Using position as a measurement leads to faster filter convergence than using velocity. (×)

10. Dual-position fine alignment changes the system's physical configuration, turning the filter model into a time-varying system, which helps improve observability. (√)

11. In EKF, the system matrix F is the first-order partial derivative matrix obtained after performing a Taylor expansion on the nonlinear system function. (√)

12. State transformation does not change the physical nature of the system, only its mathematical description. (√)

13. The ST-EKF model is not suitable for stationary base alignment scenarios. (×)

14. State transformation can improve the filter's tolerance to model uncertainty. (√)

15. The state transition matrix F in EKF is a constant matrix that does not change over time. (×)

16. Inertial navigation systems can maintain high-precision navigation for long periods without GNSS signals. (×)

17. Loosely coupled INS/GNSS systems cannot continue to provide navigation solutions after GNSS signal loss. (×)

18. Deeply coupled systems can inject INS aiding information into the tracking loops of the GNSS receiver. (√)

19. Wheeled odometers can directly measure the vehicle's lateral velocity. (×)

20. Odometry calibration can eliminate all its measurement errors. (×)

21. Odometry velocity can be used to replace acceleration integration in inertial navigation to reduce error. (√)

22. In the EKF observation equation, the velocity measurement is typically the difference between the INS-calculated velocity and the odometry velocity. (√)

23. DVL can provide absolute position information in deep-sea environments. (×)

24. USBL systems can achieve 3D positioning of AUVs underwater. (√)

25. EKF can directly process USBL slant range and azimuth angle observations through a nonlinear observation model. (√)

26. For AUVs during long underwater voyages, high-precision navigation can be maintained using INS alone. (×)

27. USBL systems can completely replace the function of DVL. (×)

28. The FEJ technique can effectively alleviate linearization inconsistency issues in EKF. (√)

Fill-in-the-blank Questions

1. DVL utilizes the (Doppler) effect to measure the vehicle's velocity relative to the seabed.

2. In the gyrocompass method, if the latitude increases, the observation sensitivity of the east-axis gyro (decreases) (fills in "increases" or "decreases").